# Setup Instructions for Raspberry Pi Kiosk

Contents

Prerequisite:	2

Initial Setup:	2

Enable SSH:	2

Change default user and password:	2

Change to RSA SSH Certificates:	3

Install Required Packages:	3

Set Auto Login:	3

Setup Wifi:	4

Update to Boot Chrome:	5

Update Network Priority	6

Create file with a list of all Pi’s	6

Update Look & Feel:	7

Disable Unused Hardware Features:	7

Setup Firewall and block everything but 443:	7

Create Read-only Image:	8

Appendix:  Current digital signage displays	10

Digital Signage with Ticket Numbers	10

Digital Signage Calling by Name	11

Digital Signage with no reception	11

# Prerequisite:

Download latest Raspbian Lite distribution (Jessie in my case) from the official Raspberry website.*

Then Install image on USB card and startup Raspberry Pi

# Initial Setup:

Login into the Raspberry pi using default User Id & Password

## Enable SSH:

Enter command: sudo raspi-config

Go to: interface Options --> Enable SSH`

Exit and  Reboot

From your workstation, log into the Raspberry Pi using SSH

## Change default user and password:

Enter Command: sudo passwd

Change password to something else.

Add another base id:

Enter Command: sudo useradd -m <newuser>l -G sudo

Enter Command: sudo passwd <newuser>

Enter Command: sudo shutdown -r now

Re-login with <newuser>

Enter Command: sudo deluser -remove-home pi

Enter Command: sudo nano /etc/sudoers.d/010_pi-nopasswd

Update this line:

pi ALL=(ALL) PASSWD: ALL

Enter Command: sudo chsh -s /bin/bash <newuser>

sbcspecial  sudo usermod -a -G adm,dialout,cdrom,audio,plugdev,games,users,input,netdev,gpio,i2c,spi <newuser>

# Change to RSA SSH Certificates:

Enter Command: cd ~

Enter Command: install -d -m 700 ~/.ssh

Enter Command: sudo nano .ssh/authorized_keys

Paste public SSH Key in file, then save and exit.

Enter Command: sudo chmod 644 ~/.ssh/authorized_keys

Enter Command: sudo chown <newuser>:<newuser> ~/.ssh/authorized_keys

Enter Command: sudo chmod 700 ~/.ssh

Enter Command: sudo nano /etc/ssh/sshd_config and add the following to end of the file:

Port <newport>

UsePAM no

PermitRootLogin no

AllowUsers <newuser>

PubkeyAuthentication yes

AuthorizedKeysFile .ssh/authorized_keys

PasswordAuthentication no

Save, exit and reboot

# Install Required Packages:

Enter Command: sudo apt-get update

Enter Command: sudo apt-get dist-upgrade

Enter Command: sudo apt-get install --no-install-recommends openbox xserver-xorg xserver-xorg-legacy xinit rpi-chromium-mods python-sense-emu python3-sense-emu unclutter matchbox-window-manager x11-xserver-utils libgl1-mesa-dri mesa-vdpau-drivers ufw libgtk-3-0 xcompmgr mesa-utils ifplugd

# Set Auto Login:

Enter Command: sudo raspi-config

Set localization (keyboard, TimeZone and system locale)

Set boot options --> b1 --> b2

Set Network options -->n3 (disasable predictive network interface)

Set Advanced Option -> Memory Split to 256MB for GPU

Set Advanced Option -> GL Driver -> G1

Reboot

# Setup Wifi:

Enter Command: sudo nano /etc/network/interfaces

Copy the following at the end of the file:

auto lo iface lo inet loopback auto eth0 allow-hotplug eth0 iface eth0 inet dhcp auto wlan0 iface wlan0 inet dhcp pre-up wpa_supplicant -B -Dwext -i wlan0 -c/boot/etc/config.conf post-down killall -q wpa_supplicant iface default inet dhcp

Save & exit.

Enter Command: sudo mkdir /boot/etc

Enter Command: sudo nano /boot/etc/config.conf and paste the following:

ctrl_interface=DIR=/var/run/wpa_supplicant GROUP=netdev #update_config=1 network={ ssid="<SSID OF Wireless Access Point>" scan_ssid=1 key_mgmt=WPA-EAP pairwise=CCMP TKIP group=CCMP TKIP eap=PEAP identity="<Active Directory User ID>" password=hash:<hashed Password> }

Save and exit

# Update to boot automatically to chromium browser:

Enter Command: sudo nano /etc/X11/Xwrapper.config

Add these lines:

allowed_users=anybody
needs_root_rights=yes

Save & Exit

Enter Command: sudo  gpasswd -a <newuser> tty

Enter Command: sudo sed -i '/^exit 0/c\chmod g+rw /dev/tty?\nexit 0' /etc/rc.local

Enter Command: sudo nano /home/<newuser>/startkiosk.sh

Paste the following:

| #!/bin/bash # disable DPMS (Energy Star) features. xset -dpms # disable screen saver xset s off # don't blank the video device xset s noblank # disable mouse pointer unclutter & # run window manager matchbox-window-manager -use_cursor no -use_titlebar no & # run browser echo "Starting chromium in 5 seconds..." sleep 5 #Get MAC ADDRESSS# IFACE=e* read LOCALMAC </sys/class/net/$IFACE/address #Set address correctly based on MAC Address while read mac tzpi url remainder do if [[ $LOCALMAC == $mac ]]; then TZPI=$tzpi; URL=$url; fi done < /boot/sites.txt #Run Browser in Kiosk Mode while : do chromium-browser %U --kiosk ${URL}?tz=${TZPI} done |  |
| --- | --- |

Save and Exit

Enter Command: sudo nano /etc/chromium-browser/customizations/00-rpi-vars

Add this line:

CHROMIUM_FLAGS="--ignore-gpu-blacklist --force-gpu-rasterization --enable-zero-copy --enable-native-gpu-memory-buffers --disable-infobars --disable-session-crashed-bubble --noerrdialogs --incognito --kiosk"

Save & Exit

Enter Command: sudo chmod +x /home/<newuser>/startkiosk.sh

Enter Command: sudo nano /home/<newuser>/.bashrc

Add the following to the end of the file:

if [ -z "${SSH_TTY}" ]; then

xinit ~/startkiosk.sh

fi

Save & Exit

# Update Network Priority

Enter Command: sudo nano /etc/ifplugd/action.d/ifupdown

Add the two lines in the following file that are highlighted yellow:

#!/bin/sh set -e case "$2" in up) /sbin/ifup $1 if [ "$1" == eth0 ]; then /sbin/ifdown wlan0 ; fi # This is a new bit ;; down) /sbin/ifdown $1 if [ "$1" == eth0 ]; then /sbin/ifup wlan0 ; fi # Another new bit ;; esac

Save & Exit & Reboot

# Create file with a list of all Pi’s

Enter Command: sudo nano /boot/sites.txt

Use this template:

| Mac_Address | Time_Zone | URL | Location | WIFI_MAC | Static_IP |
| --- | --- | --- | --- | --- | --- |
| <ethernet_mac> | America/Vancouver | <URL>/smartboard/<Office_id> | TestMachine | <wifi mac> | <Reserved IP> |
| <ethernet_mac> | America/Vancouver | <URL>/smartboard/<Office_id> | TestMachine | <wifi mac> | <Reserved IP> |

Save & Exit

# Update Look & Feel:

Enter Command: sudo nano /etc/systemd/timesyncd.conf

Update with your internal time servers:

NTP=<time server 1> <time server 2>

Enter Command: sudo nano /etc/rc.local & add this before 'exit 0':

#Suppress Kernel Messages

dmesg --console-off

Enter Command: sudo nano /etc/systemd/system/autologin\@.service

Change your auto login ExecStart

FROM: ExecStart=-/sbin/agetty --autologin pi --noclear %I $TERM

TO: ExecStart=-/sbin/agetty --skip-login --noclear --noissue --login-options "-f <newuser>" %I $TERM

# Disable Unused Hardware Features:

Enter Command: sudo nano /boot/config.txt

Add this line:

dtoverlay=pi3-disable-bt

Save & Exit

Enter Command: sudo nano /etc/modprobe.d/raspi-blacklist.conf

Add these lines:

blacklist btbcm

blacklist hci_uart

Save & Exit

Enter Command: sudo systemctl disable hciuart

# Setup Firewall and block everything but 443:

Enter Command: sudo ufw default deny incoming

Enter Command: sudo ufw default allow outgoing

Enter Command: sudo ufw allow 443

Enter Command: sudo ufw allow <newport>

Enter Command: sudo ufw enable

Enter Command: sudo ufw -- force enable

# Create Read-only Image:

Enter Command: sudo bash

Enter Command: rm -r /var/log

Enter Command: sudo nano /etc/rsyslog.conf

Comment out all items in Rules Section and add this line:

*.*     ~

Enter Command: echo overlay >>/etc/initramfs-tools/modules

Enter Command: nano /etc/initramfs-tools/scripts/overlay

Paste the following:

# Local filesystem mounting -*- shell-script -*- # # This script overrides local_mount_root() in /scripts/local # and mounts root as a read-only filesystem with a temporary (rw) # overlay filesystem. # . /scripts/local local_mount_root() { local_top local_device_setup "${ROOT}" "root file system" ROOT="${DEV}" # Get the root filesystem type if not set if [ -z "${ROOTFSTYPE}" ]; then FSTYPE=$(get_fstype "${ROOT}") else FSTYPE=${ROOTFSTYPE} fi local_premount # CHANGES TO THE ORIGINAL FUNCTION BEGIN HERE # N.B. this code still lacks error checking modprobe ${FSTYPE} checkfs ${ROOT} root "${FSTYPE}" # Create directories for root and the overlay mkdir /lower /upper # Mount read-only root to /lower if [ "${FSTYPE}" != "unknown" ]; then mount -r -t ${FSTYPE} ${ROOTFLAGS} ${ROOT} /lower else mount -r ${ROOTFLAGS} ${ROOT} /lower fi modprobe overlay # Mount a tmpfs for the overlay in /upper mount -t tmpfs tmpfs /upper mkdir /upper/data /upper/work # Mount the final overlay-root in $rootmnt mount -t overlay \ -olowerdir=/lower,upperdir=/upper/data,workdir=/upper/work \ overlay ${rootmnt} }

Save and exit.

Enter Command: update-initramfs -c -k $(uname -r)  * Will create initrd.img<kernelversion>

Enter Command: cd /boot

Enter Command: mv initrd.img-<kernelversion> initrd7.img

Enter Command: nano /boot/config.txt

At the end, add the following.

disable_splash=1

kernel=kernel7.img

initramfs initrd7.img

Save & Exit

Enter Command: sudo nano /boot/cmdline.txt

Add the following at the beginning of the same line:

boot=overlay console=tty3 quiet splash loglevel=0 logo.nologo vt.global_cursor_default=0 net.ifnames=0

Save & Exit

Reboot

You should now have a read-only Image that will connect to Wifi & Display Chromium with the assigned URL.

# Appendix:  Current digital signage displays

# Digital Signage with Ticket Numbers

# Digital Signage Calling by Name

# Digital Signage with no reception